Session 12 — Problem B: LED by Timer-Paced DMA on STM32F103

Three API stages (HAL / CMSIS / raw DSL) — one register fingerprint, verified on board

Table of Contents

# 1. Introduction

Problem A showed DMA as a conveyor belt: declare a transfer, press enable, and the hardware copies with the CPU standing aside. But a belt that dumps everything at bus speed is only half the story — most real DMA work is paced by events. A peripheral raises a *DMA request* each time it is ready, and the DMA controller moves exactly one element per request. That is the belt with a metronome.

Why event-paced DMA matters. It is the mechanism behind virtually every periodic I/O pattern in embedded systems: a timer beats, and on every beat one sample moves — no interrupt handler, no CPU wake-up, perfect timing jitter-free at the hardware level:

- Waveform generation — a timer paces DAC (or GPIO) writes from a RAM pattern: audio tones, arbitrary signal generators, WS2812 LED-strip bitstreams.

- PWM pattern engines — timer update events stream new duty-cycle values into CCR registers (motor ramps, LED dimming curves).

- Display scanning — LED-matrix and 7-segment multiplexing at a fixed refresh rate with zero CPU load.

- Periodic sampling — the mirror image: a timer paces ADC conversions into a ring buffer.

Problem B (this report): the smallest possible instance of that pattern on the STM32F103 — blink the on-board LED (PC13) with the CPU fully halted. TIM1 overflows every 400 ms and raises a DMA request (not an interrupt); DMA1 Channel 5 answers by writing the next word of a 2-word pattern into GPIOC_ODR; circular mode reloads the counter forever. The LED blinks even while the debugger holds the core at a breakpoint — the strongest possible proof that the CPU is not involved.

Figure 1. Problem B datapath: TIM1 update event → DMA1 Channel 5 (circular) → GPIOC_ODR → LED PC13; the CPU does not interfere — the transfer is paced by the timer, not by code

# 2. Problem Identification

Problem A left two questions deliberately open:

- Nobody saw the belt move. mem2mem rips through at bus speed (<1 µs), so every

halt already showed CNDTR = 0. To *watch* DMA work — the counter stepping down, the destination changing — the transfer must be slowed to human speed by an external pacer.

- The peripheral-request side was untouched. Problem A used MEM2MEM, which needs

no request line. Real DMA is driven by peripheral requests routed through a fixed channel mapping (RM0008 Table 78: TIM1_UP is hard-wired to DMA1 Channel 5) and gated by request-enable bits (TIM1_DIER.UDE) — a whole mechanism absent from Problem A.

Problem B closes both gaps, and adds two new register-level concepts on the way:

circular mode (CIRC = 1: CNDTR self-reloads, the transfer never "completes") and memory→peripheral direction with a fixed destination (PINC = 0).

Problem B spans three stages — the same problem re-implemented at three API layers, all covered by this report:

| Stage | API layer | Status |
| --- | --- | --- |
| 0_1 | HAL (vendor functions + vendor names) | PASS on board |
| 0_2 | CMSIS (vendor names only, zero vendor code) | PASS on board |
| 0_3 | raw registers (project DSL per STANDARD.md — nothing vendor) | PASS on board |

Stages 0_2 and 0_3 are support stages of 0_1: their purpose is to expose which components of the vendor API stage 0_1 actually used, and to demonstrate the extraction technique — read the vendor source, keep only the register writes, re-verify on board.

Problem A — the mem-to-mem sibling problem of Session 12 — has its own report; the two share only the session map (section 4.1) and the acceptance method.

# 3. Objective

Acceptance criteria set before writing any code:

- Build: iarbuild on config DMA_B12_S01_HAL (with PROBLEM_B selected) completes with 0 errors, 0 warnings.

- Function: LED PC13 blinks with a 400 ms half-period — and keeps blinking while the core is halted at a breakpoint (the "CPU not involved" proof).

- Visible countdown: DMA1_CH5.CNDTR can be watched stepping 2 → 1 → 2 in Live Watch — the observation Problem A could not offer.

- Correct pacing chain in registers: TIM1_DIER.UDE = 1 (DMA request, *not* UIE interrupt), TIM1_CR1.CEN = 1, PSC = 7999, ARR = 399.

- Stage-0 rule respected: no polling loop, no interrupt enabled; the CPU contributes nothing after start-up.

- Minimal vendor footprint: compile only the HAL modules strictly required (target: 3 of ≈30 — DMA, TIM, GPIO).

- Evidence captured, not recalled: annotated C-SPY screenshots in Problem_B/assets/; the register set {CCR, CNDTR, CPAR, CMAR} + {CR1, DIER, PSC, ARR} becomes the fingerprint that stages 0_2 and 0_3 must reproduce.

# 4. Methodology and Progress

## 4.1 Method — one problem, three API rungs, one fingerprint

Problem B walks the same priority ladder as Problem A: solve at HAL (rung 0_1), re-solve at CMSIS (rung 0_2), re-solve at raw registers (rung 0_3), then diff the register fingerprints — if all three leave the same values in the DMA channel and TIM1 registers, they are provably the same thing at the hardware level.

Figure 2. The Session 12 API ladder: the application descends through HAL / CMSIS / raw registers to the hardware; the register bank on the left shows which DMA registers the session touches (green = written, yellow = read); the arrow marks increasing hardware control and precision

Problem B's architecture — the one datapath all three stages must reproduce:

Problem B architecture (shared by stages 0_1 / 0_2 / 0_3)

g_led_pattern[2] @ 0x20000000            GPIOC_ODR @ 0x4001100C

+------------------------+   DMA1 Channel 5   +---------------+     PC13

| RAM: { 0x2000, 0x0 }   | =================> | GPIO port C   | --> LED

+------------------------+  MEM->PERIPH, word +---------------+  (active-low)

^               MINC=1, PINC=0

|               CIRCULAR, N=2

one word per beat

|

+------------------------+

|  TIM1 update event     |   PSC=7999, ARR=399

|  DIER.UDE=1 (request,  |   8 MHz HSI / 8000 / 400

|  NOT an interrupt)     |   -> one beat every 400 ms

+------------------------+

CPU: while(1) { }   -- no polling, no interrupts (Stage-0 rule)

This report covers the whole Problem B row of the session map below. The matrix is now complete — all six cells PASS on board; only the application layer remains.

Figure 3. The Session 12 map, final state: two problems × three API rungs, 6/6 cells PASS on board with the same register fingerprint per problem; the dashed application layer is the only piece left

## 4.2 Toolchain and build profiles

Hardware and toolchain are identical to Problem A: STM32F103C8T6 (Blue Pill), ST-Link V2 over pure SWD, C-SPY in IAR EWARM 8.3. Problem B reuses the **same build profile

DMA_B12_S01_HAL (`STM32F103xB` + `USE_HAL_DRIVER`). Each (problem × stage) cell is one self-contained file living beside the hand library's own `source/main.c` — this problem's trio is `main_b12_B_S01.c` / `_S02.c` / `_S03.c`. The mains are *our* application code; stages that need HAL/CMSIS *call into* the vendor warehouse (`Manufacturer_Package/`) through include paths. The cell to run is selected by keeping exactly one** main_b12_*.c included in the build. Two more vendor modules join the compile list for this problem: stm32f1xx_hal_gpio.c (pin setup) and stm32f1xx_hal_tim.c (timer base), alongside the stm32f1xx_hal_dma.c already wired for Problem A.

Adding / removing a flag (define). In the IAR GUI: *Project → Options… → C/C++ Compiler → Preprocessor → Defined symbols* — one symbol per line, set per configuration (select the profile in the workspace drop-down first). Adding a line defines the flag for that profile only; deleting the line removes it. The same list is stored per configuration as the <CCDefines> option inside test.ewp, which is why a profile's flag changes show up as reviewable diffs in git.

Figure 4. IAR configuration — debug profile setup: one project fans out to five build profiles (only the enabled flags differ; the legend decodes each flag); the hatched profile (Debug_Vendor) is kept in the project but no longer used; all profiles feed iarbuild — PASS flashes to the board, FAIL loops back to fix-and-rebuild

## 4.3 Stage 0_1 — HAL implementation (evidence in code)

The pattern is two whole-word ODR images, replayed forever by circular DMA —

[0] = bit 13 set (PC13 high, LED off: the Blue Pill LED is active-low),

[1] = all clear (PC13 low, LED on):

static uint32_t g_led_pattern[2] = { (1uL << 13), 0uL };

The complete Stage 0_1 core — timer as metronome, DMA as belt, one hook between them:

static void Stage01_LedByTimDma(void)

{

/* -- timer: 8 MHz HSI / 8000 = 1 kHz tick, reload every 400 ticks -- */

__HAL_RCC_TIM1_CLK_ENABLE();

htim1.Instance               = TIM1;

htim1.Init.Prescaler         = 8000u - 1u;  /* 1 tick = 1 ms            */

htim1.Init.Period            = 400u - 1u;   /* update event each 400 ms */

htim1.Init.CounterMode       = TIM_COUNTERMODE_UP;

htim1.Init.RepetitionCounter = 0u;          /* TIM1: fire every reload  */

HAL_TIM_Base_Init(&htim1);                  /* writes PSC/ARR/CR1       */

/* -- DMA: memory (pattern) -> peripheral (ODR), circular, word cells -- */

__HAL_RCC_DMA1_CLK_ENABLE();

hdma.Instance                 = DMA1_Channel5;   /* TIM1_UP lives here  */

hdma.Init.Direction           = DMA_MEMORY_TO_PERIPH;

hdma.Init.PeriphInc           = DMA_PINC_DISABLE;   /* ODR never moves  */

hdma.Init.MemInc              = DMA_MINC_ENABLE;    /* walk the pattern */

hdma.Init.PeriphDataAlignment = DMA_PDATAALIGN_WORD;

hdma.Init.MemDataAlignment    = DMA_MDATAALIGN_WORD;

hdma.Init.Mode                = DMA_CIRCULAR;    /* never "done": no TC  */

HAL_DMA_Init(&hdma);

HAL_DMA_Start(&hdma, (uint32_t)g_led_pattern, (uint32_t)&GPIOC->ODR, 2u);

/* -- hook them together and press play -- */

__HAL_TIM_ENABLE_DMA(&htim1, TIM_DMA_UPDATE);   /* DIER.UDE = 1         */

HAL_TIM_Base_Start(&htim1);                     /* CR1.CEN  = 1 - GO    */

/* NO poll, NO interrupt: from here the LED blinks with the CPU halted. */

}

One debugging trick worth recording — needed at stages 0_1/0_2 only:

DMA1_Channel5 and TIM1 are preprocessor macros, invisible to C-SPY Live Watch. Two

__root pointer constants make the whole register structs watchable (*g_dbg_dma5,

*g_dbg_tim1) without being dead-stripped. Stage 0_3 deletes both pointers — see 4.5 for why they become unnecessary:

__root DMA_Channel_TypeDef * const g_dbg_dma5 = DMA1_Channel5;

__root TIM_TypeDef         * const g_dbg_tim1 = TIM1;

## 4.4 Stage 0_2 — CMSIS: extracting the API

The extraction technique: read the vendor source, keep only what it writes. Tracing

stm32f1xx_hal_gpio.c / _tim.c / _dma.c shows the five HAL calls and two macros of 4.3 collapse to ~15 register writes hiding under ~900 lines of vendor code. Two of them are worth calling out: the GPIO leg is just *one* CRH nibble write after the clock enable, and HAL_TIM_Base_Init secretly ends with EGR = UG — PSC is a *preload* register, and without that forced update event the first period would run at the old prescaler. Stage 0_2 reproduces both faithfully:

#include "stm32f103xb.h"     /* CMSIS device header: names only, no code */

static void Stage02_Led_Gpio_Init(void)

{

RCC->APB2ENR |= RCC_APB2ENR_IOPCEN;       /* was __HAL_RCC_GPIOC_CLK_ENABLE */

GPIOC->CRH = (GPIOC->CRH

& ~(GPIO_CRH_MODE13 | GPIO_CRH_CNF13))  /* wipe pin-13 nibble   */

|   GPIO_CRH_MODE13_1;         /* was HAL_GPIO_Init: MODE13=10  */

}                                             /* (2 MHz), CNF13=00 = OUTPUT_PP */

static void Stage02_LedByTimDma(void)

{

RCC->APB2ENR |= RCC_APB2ENR_TIM1EN;       /* was __HAL_RCC_TIM1_CLK_ENABLE */

TIM1->ARR = 400u - 1u;                    /* was Init.Period               */

TIM1->PSC = 8000u - 1u;                   /* was Init.Prescaler            */

TIM1->RCR = 0u;                           /* was Init.RepetitionCounter    */

TIM1->EGR = TIM_EGR_UG;                   /* HAL tail: latch PSC NOW       */

RCC->AHBENR |= RCC_AHBENR_DMA1EN;         /* was __HAL_RCC_DMA1_CLK_ENABLE */

DMA1_Channel5->CCR &= ~DMA_CCR_EN;        /* config only while disabled    */

DMA1_Channel5->CCR = DMA_CCR_DIR | DMA_CCR_CIRC | DMA_CCR_MINC

| DMA_CCR_MSIZE_1 | DMA_CCR_PSIZE_1;  /* was HAL_DMA_Init */

DMA1_Channel5->CNDTR = 2u;                /* was HAL_DMA_Start count arg   */

DMA1_Channel5->CPAR  = (uint32_t)&GPIOC->ODR;  /* dest  (DIR=1: CPAR=dst)  */

DMA1_Channel5->CMAR  = (uint32_t)g_led_pattern;/* source                   */

DMA1_Channel5->CCR  |= DMA_CCR_EN;        /* armed -> CCR = 0x00000AB1     */

TIM1->DIER |= TIM_DIER_UDE;               /* was __HAL_TIM_ENABLE_DMA      */

TIM1->CR1  |= TIM_CR1_CEN;                /* was HAL_TIM_Base_Start - GO   */

}

## 4.5 Stage 0_3 — raw DSL: owning the names, and simpler debugging

The final stage removes the vendor's *vocabulary* too. Problem B pulls four hand-written maps: the new dma.h (written for Problem A's 0_3, reused here — its channel array makes CH[4] = the manual's channel 5) plus the library's existing

advanced_timer.h, gpio.h, rcc.h. All are placed instances in stm32f103c8t6.h:

#include <stm32f103c8t6.h>   /* hand-written map: RM0008 only, zero vendor */

static void Stage03_Led_Gpio_Init(void)

{

RCC.APB2_ENR.BITS.IOPC = 1;         /* clock first, always               */

GPIOC.CRH.BITS.MODE_13 = 2;         /* 2 = output, 2 MHz max             */

GPIOC.CRH.BITS.CNF_13  = 0;         /* 0 = general-purpose push-pull     */

}

static void Stage03_LedByTimDma(void)

{

RCC.APB2_ENR.BITS.TIM1 = 1;

TIM1.ARR = 400u - 1u;               /* update event each 400 ms          */

TIM1.PSC = 8000u - 1u;              /* 1 tick = 1 ms                     */

TIM1.RCR = 0u;

TIM1.EGR.BITS.UG = 1;               /* latch PSC now (0_2 lesson)        */

RCC.AHB_ENR.BITS.DMA1 = 1;

DMA1.CH[4].CCR.REG = 0;             /* CH[4] = RM0008 channel 5          */

DMA1.CH[4].CCR.BITS.DIR   = 1;      /* read memory, write peripheral     */

DMA1.CH[4].CCR.BITS.CIRC  = 1;      /* CNDTR auto-reloads forever        */

DMA1.CH[4].CCR.BITS.MINC  = 1;

DMA1.CH[4].CCR.BITS.MSIZE = 2;      /* 32-bit cells (PINC stays 0)       */

DMA1.CH[4].CCR.BITS.PSIZE = 2;

DMA1.CH[4].CNDTR.BITS.NDT = 2u;

DMA1.CH[4].CPAR = (uint32_t)&GPIOC.ODR;

DMA1.CH[4].CMAR = (uint32_t)g_led_pattern;

DMA1.CH[4].CCR.BITS.EN = 1;         /* armed -> CCR = 0x00000AB1         */

TIM1.DIER.BITS.UDE = 1;             /* DMA request on update - NOT IRQ   */

TIM1.CR1.BITS.CEN  = 1;             /* GO                                */

}

Checking registers in Live Watch gets *simpler* at this stage — the two `__root` debug pointers of 4.3 are deleted. The vendor's TIM1 was a cast macro (#define TIM1 ((TIM_TypeDef *)0x40012C00)) — no symbol exists, so C-SPY needed the

g_dbg_* pointer detour. The hand map instead *places real variables* (__root __no_init volatile ADVANCED_TIMER_TypeDef TIM1 @ 0x40012C00; in

stm32f103c8t6.h), so the debugger knows them by name. The whole check procedure is: type `DMA1`, `TIM1`, `GPIOC` directly into Live Watch, expand, and read fields by name — DMA1.CH[4].CNDTR.BITS.NDT bouncing 2 ↔ 1, TIM1.DIER.BITS.UDE = 1,

GPIOC.ODR.REG flipping 0x2000 ↔ 0x0000 — no pointer dereference, no per-register watch entries.

## 4.6 Verification method

- Flash via ST-Link, run through start-up, then halt the core — the LED must keep

blinking (hardware-only proof).

- Watch the belt move: in Live Watch — via *g_dbg_dma5 / *g_dbg_tim1 at stages

0_1/0_2, or directly by name (DMA1, TIM1) at stage 0_3 — CNDTR steps

2 → 1 → 2 → … one step per 400 ms beat while CNT ticks.

- Register fingerprint: capture DMA1_CH5 {CCR, CNDTR, CPAR, CMAR} and TIM1

{CR1, DIER, CNT, PSC, ARR}; screenshot with annotations (red = live values, blue = configuration values).

- Repeat per stage and diff the fingerprints: 0_1 recorded the reference; 0_2 and

0_3 must reproduce it bit-for-bit (they did — CCR5 = 0x0AB1, DIER = 0x100,

PSC = 7999, ARR = 399, LED blinking with the core halted, at all three stages).

## 4.7 Progress

| Step | Outcome |
| --- | --- |
| Write the Stage 0_1 main (now main_b12_B_S01.c) | done |
| Register hal_gpio.c + hal_tim.c in test.ewp | done — other configs untouched |
| Build (DMA_B12_S01_HAL) | 0 errors, 0 warnings |
| On-board run: LED blinks with core halted | PASS |
| CNDTR countdown observed in Live Watch | PASS — screenshots in Problem_B/assets/ |
| Stage 0_2: trace HAL → CMSIS writes (GPIO + TIM + DMA) | done — build 0 err / 0 warn |
| On-board run (0_2) | PASS — same fingerprint as 0_1 |
| Stage 0_3: reuse dma.h + existing TIM/GPIO/RCC maps; drop debug pointers | done — build 0 err / 0 warn |
| On-board run (0_3) | PASS — same fingerprint; Problem B row complete |

# 5. Result and Evaluation

## 5.1 Register fingerprint — DMA1 · Channel 5 + TIM1

In both screenshots below, red boxes mark live/changing values (CNDTR, CNT — the proof the metronome is beating) and blue boxes mark configuration/fingerprint values (the proof it was set up as coded).

Table 1 — the fingerprint after start-up (read in C-SPY via *g_dbg_dma5 / *g_dbg_tim1):

| Register | Value | Meaning |
| --- | --- | --- |
| DMA1_CCR5 | 0x00000AB1 | channel configuration (decoded in Table 2) |
| DMA1_CNDTR5 | 2 ↔ 1 (live) | words remaining — reloads to 2 (circular) |
| DMA1_CPAR5 | 0x4001100C | destination = &GPIOC->ODR (fixed, PINC=0) |
| DMA1_CMAR5 | 0x20000000 | source = g_led_pattern[0] |
| TIM1_CR1 | 0x00000001 | CEN = 1 — counter running |
| TIM1_DIER | 0x00000100 | UDE = 1 — update-DMA request; UIE = 0 (no interrupt) |
| TIM1_CNT | 300 / 341 (live) | counting 0…399, one lap = 400 ms |
| TIM1_PSC | 7999 | 8 MHz / 8000 = 1 kHz tick |
| TIM1_ARR | 399 | update event every 400 ticks = 400 ms |

Table 2 — decoding CCR5 = 0x00000AB1 (contrast with Problem A's 0x00004AC1):

| Bit-field | Value | Meaning |
| --- | --- | --- |
| MEM2MEM | 0 | not memory→memory — a real peripheral request drives it |
| DIR | 1 | read from memory, write to peripheral |
| CIRC | 1 | circular — CNDTR self-reloads, transfer never "completes" |
| MSIZE / PSIZE | 0b10 / 0b10 | 32-bit (word) elements on both sides |
| MINC / PINC | 1 / 0 | pattern pointer walks; ODR address stays fixed |
| TCIE / HTIE / TEIE | 0 | all interrupts off (per the Stage 0 rule) |
| EN | 1 | channel enabled |

→ a 100% match with hdma.Init / htim1.Init in the code of section 4.3.

Figure 5. First capture: DMA1 channel 5 mid-pattern — CNDTR = 1 (red, one word left before reload), CCR = 0x0AB1 / CPAR = 0x4001100C / CMAR = 0x20000000 (blue); below, TIM1 with CR1.CEN = 1, DIER = 0x100 (UDE), CNT = 300 ticking (red), PSC = 7999, ARR = 399 (blue)

Figure 6. Second capture, moments later: CNDTR has reloaded to 2 (red) — the circular belt restarted by itself; TIM1 CNT = 341 (red) shows the metronome still ticking; every blue configuration value is unchanged

## 5.2 The decisive observation

Two captures, taken seconds apart, tell the whole story:

- `CNDTR` moved `1 → 2` — it hit zero and *reloaded itself*: circular mode working, one word per timer beat. This is the countdown Problem A was too fast to show.

- `CNT` moved `300 → 341` — the metronome ticks on independently.

- Every configuration register is bit-identical between captures — nothing but the two live counters changes, because nothing but the hardware is running: the core was halted the entire time, and the LED kept blinking.

Figure 7. Timing reconstruction of the observation: TIM1 beats every 400 ms (orange = update event, a DMA request via DIER.UDE — no interrupt); CNDTR steps 2 → 1 and self-reloads when it hits 0 (CIRC = 1); ODR bit 13 toggles the active-low LED — all inside the gray band where the core sits halted at a breakpoint, executing nothing

## 5.3 Evaluation against the objectives

| Objective (section 3) | Verdict |
| --- | --- |
| Build 0 errors / 0 warnings | met |
| LED blinks at 400 ms half-period with core halted | met (on-board run) |
| CNDTR countdown visible, 2 → 1 → 2 | met (screenshots, section 5.1) |
| Pacing chain: UDE=1, UIE=0, CEN=1, PSC=7999, ARR=399 | met (Table 1) |
| No polling, no interrupts | met (DIER.UIE = 0, TCIE/HTIE/TEIE = 0, empty while(1)) |
| Minimal vendor footprint (3/≈30 modules) | met — DMA + TIM + GPIO only (section 5.4) |
| Evidence captured; fingerprint recorded for 0_2 / 0_3 | met (Tables 1–2 are the reference) |

## 5.4 Vendor-API footprint — stage by stage

The vendor supplies two separable things: functions (behavior — code that runs) and

names (vocabulary — #defines and structs). Each stage strips one of them.

Stage 0_1 — HAL: functions + names. Problem B needs three HAL modules instead of one — stm32f1xx_hal_dma.c, stm32f1xx_hal_tim.c, stm32f1xx_hal_gpio.c (3 of ≈30, ~10%) — and five public functions plus two macros: HAL_GPIO_Init, HAL_TIM_Base_Init,

HAL_TIM_Base_Start, HAL_DMA_Init, HAL_DMA_Start, __HAL_TIM_ENABLE_DMA,

__HAL_RCC_*_CLK_ENABLE. Still an ignition switch, not a dependency: ~900 vendor lines run for what is really ~15 register writes.

Figure 8. Vendor API usage in Problem B at stage 0_1, with the two counting units defined by example: 1 module = 1 vendor driver file (3 of 30 compiled: hal_dma + hal_tim + hal_gpio, ~10%); 1 function = 1 public API inside those files (5 of 94 called: HAL_GPIO_Init, HAL_TIM_Base_Init/Start, HAL_DMA_Init/Start, ~5%); green = used, gray = stripped by the linker

Stage 0_2 — CMSIS: names only. Vendor functions drop to zero — the ~15 writes are hand-written (section 4.4). About 21 vendor names remain, all from stm32f103xb.h: peripheral macros (TIM1, GPIOC, DMA1_Channel5), bit masks (TIM_DIER_UDE,

TIM_CR1_CEN, TIM_EGR_UG, GPIO_CRH_MODE13, the DMA_CCR_* family, the

RCC_*ENR_* enables) and two struct typedefs — vocabulary with zero behavior behind it.

Stage 0_3 — raw DSL: nothing vendor. The names now come from four hand-written maps (dma.h — new this session — plus advanced_timer.h, gpio.h, rcc.h), the behavior from the same ~15 writes. Vendor footprint: 0 modules, 0 functions, 0 names — while the board fingerprint stays bit-identical (CCR5 = 0x0AB1, DIER = 0x100,

PSC = 7999, ARR = 399) and debugging gets *simpler* (real placed symbols, no

__root helper pointers).

Figure 9. Vendor-API footprint of Problem B across the three stages: the vendor's FUNCTIONS row is stripped at 0_2 (traced into 15 hand-written writes) and its NAMES row at 0_3 (own maps: dma.h + advanced_timer.h + gpio.h + rcc.h); orange = vendor material still in the build, green check = replaced by a hand-written equivalent — the fingerprint stays 0x0AB1 / 0x100 / 7999 / 399 throughout

## 5.5 Lessons learned

- A DMA request is not an interrupt. TIM1_DIER has two separate enable bits:

UIE (update *interrupt* → CPU runs a handler) and UDE (update *DMA request* → hardware moves data, CPU untouched). Problem B sets only UDE — the LED blinks with zero software execution.

- DMA channels are hard-wired to peripherals. TIM1_UP can only arrive on DMA1

Channel 5 (RM0008 Table 78); the channel is chosen by looking it up, not by preference.

- Circular mode redefines "done". With CIRC = 1 there is no transfer-complete

state to wait for — CNDTR reloads forever. The Problem A habit of "check TCIF, check CNDTR == 0" simply does not apply.

- Word-writes to `ODR` rewrite the whole port. The 2-word pattern clears every

other PORTC bit each beat — harmless here (PC13 is the only pin wired), but a real application would use BSRR set/reset semantics instead.

- The Blue Pill LED is active-low: bit13 = 1 turns it *off*. Getting the pattern

order wrong costs nothing but confusion — worth one comment in the code.

- __root debug pointers make macro-peripherals watchable — but only vendor stages

need them.** DMA1_Channel5 is a cast constant, not a symbol; a __root pointer variable gives C-SPY a real object to expand, and survives the linker's dead-stripping despite being referenced by nothing. At stage 0_3 the hand-written map *places real symbols* (TIM1 @ 0x40012C00), so both pointers are deleted and Live Watch works by name: DMA1, TIM1, GPIOC.

- The build profile does not choose the stage — the source labels do. With a 0_2/0_3

label active, even the HAL profile builds the same hand-written code (its HAL modules compile but are dead-stripped). Both profiles then produce the same binary — an accidental but perfect control experiment.

# 6. References

- STMicroelectronics, *RM0008 — STM32F101xx/102xx/103xx/105xx/107xx Reference Manual*: ch. 13 "DMA controller" (Table 78 — DMA1 request mapping: TIM1_UP → Channel 5), ch. 14 "Advanced-control timers (TIM1)" (CR1, DIER.UDE, PSC, ARR).

- STMicroelectronics, *STM32F103x8/B Datasheet* — local copy: docs/datasheets/stm32f10xxx.pdf.

- STMicroelectronics, *UM1850 — Description of STM32F1 HAL and low-layer drivers* (HAL_DMA, HAL_TIM, HAL_GPIO APIs).

- Vendor driver sources: stm32f1xx_hal_dma.c, stm32f1xx_hal_tim.c, stm32f1xx_hal_gpio.c (STM32CubeF1 package, mirrored in this repository).

- Project coding standard: STANDARD.md (register-map DSL used at rung 0_3).

- Captured on-board evidence: Buoi_12/Problem_B/assets/ (fig_12_B_S01_CNDTR_countdown.png, fig_12_B_S01_CNDTR_reload.png, annotated).

- Sibling report: *Session 12 — Problem A: DMA mem-to-mem (HAL, rung 0_1)* — Buoi_12/Problem_A/docs/report_Buoi_12_Problem_A_EN.docx.

List of Figures